BRAC University

CSE341 : Microprocessors

Make Up Quiz

Duration : 25 Minutes

Total Marks : 20

ID:                                     Name: 						        Sec:

| Address | 90h | 91h | 88h | 89h | 138h | 139h | 136h | 137h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data | 24h | 20h | 01h | 01h | 25h | 20h | 01h | 00h |

The values of internal registers of an intel 8086 are as follows, CS: 8086h, IP: 2233h, SS: 4240h. Suppose it receives an interrupt signal. The starting address of an interrupt service routine is 20251h. If the segment address for the interrupt service routine is 2025h,

- Determine the interrupt type in hex. [7]

- If the 8086 decides to service the interrupt, do the bits of flag register change? Explain. [1]

- When servicing an interrupt why are IF and TF cleared? [2]

- Determine the values of CS, IP and SS at two time frames.  [6]

  - When the interrupt starts being serviced

  - After the interrupt is completed

- Explain in detail with steps of how the interrupt will be serviced by the intel 8086. [4]