CMOS, Digital Electronics, DC response

Q1.

Scenario 1:

A classroom has three temperature sensors to control the temperature of the room by controlling the air conditioner installed in that room. The temperature sensors produce a high output if the temperature crosses a certain threshold. The air conditioner turns on if the outputs of all three sensors are high, otherwise it remains off.

Scenario 2:

Figure: 1					Figure: 2

Figure 2 shows the output waveshape of the device implemented in Figure 1.

| (a) | Identify the logic circuit required to implement the system mentioned in scenario 1 and implement the circuit using 2:1 mux. | [5] |
| --- | --- | --- |
| (b) | Identify the circuit implemented in Figure 1 from the timing diagram of Figure 2. Also identify Device 1 shown in Figure 1. | [3] |
| (c) | Draw the circuit of Device 1 using transmission gates. | [3] |
| (d) | Explain why n-MOS pass transistors produce good 0 and bad 1, and p-MOS pass transistors produce bad 0 and good 1 with proper mathematical justification. | [4] |

Q2.