FSM

Q1. Kelvin is tasked with building an FSM that would detect 10 or 011 overlapping sequences coming out of a communication channel. A sample is given below:

(w indicates input sequence values and z indicates output)

| Clock cycle | t0 | t1 | t2 | t3 | t4 | t5 | t6 | t7 | t8 | t9 | t10 | t11 | t12 | t13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| z | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |

Note: Use the table below for Gray coding if needed.

| Decimal Equivalent | Binary Code |  |  | Gray Code |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | B2 | B1 | B0 | G2 | G1 | G0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 0 | 1 | 0 |
| 4 | 1 | 0 | 0 | 1 | 1 | 0 |
| 5 | 1 | 0 | 1 | 1 | 1 | 1 |
| 6 | 1 | 1 | 0 | 1 | 0 | 1 |
| 7 | 1 | 1 | 1 | 1 | 0 | 0 |

| (a) | Determine the type of FSM (Mealy/Moore) to be implemented | [1] |
| --- | --- | --- |
| (b) | Draw the state diagram. Use no more than 6 states. Using more states will result in deduction of marks. | [4] |
| (c) | Draw the state assignment table using Binary encoding. | [3] |
| (d) | Determine the expression of only the output, z. No need to draw circuit. | [2] |

Q2. Design a finite state machine (FSM) for a car alarm system. The system has two binary inputs: x, indicating the lock status of the car door (x = 1 if unlocked, x = 0 if locked) and y, indicating the door status (y = 1 if the door is open, y = 0 if closed). The output z is used to control the alarm: z = 1 when the alarm is active, and z = 0 otherwise. The alarm should be triggered (z = 1) if the door is opened (y = 1) while it is still locked (x = 0) for two consecutive clock cycles. An example is shown below:

| x | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| z | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |

| (a) | Determine the type of FSM that would be needed for the given problem and design the state diagram for the corresponding FSM. | [2] |
| --- | --- | --- |
| (b) | Derive the state-assigned table using the Gray encoding from your state diagram in (a). | [2] |
| (c) | Find the expression for the next state and the output for the FSM using KMAP. | [3] |
| (d) | Calculate the number of flip-flops required for the circuit implementation and draw the circuit for this FSM. | [3] |