Power

Q1.

TSMC are planning to design a new chip in a 1.1 V,  22 nm ( λ=10 nm) technology which has 200 million transistors, of which 25% are used for building logic gates and the rest are used for memory arrays. The average logic transistor width is 6λ and the average memory transistor width is 4λ. Other information of the device is given below

- The gate and diffusion capacitances are 1.2 fF/μm and  0.6 fF/μm

- The system clock frequency is 3.5 GHz. In every 100 clock cycles, the number of switching for the logic and memory transistors are 10 and 2 respectively.

- For both of the transistors, each transistor contributes the gate leakage, junction leakage and subthreshold leakage currents of approximately 20 pA, 15 pA and 25 pA respectively. The total short circuit power is 0.05 W. The total accepted power consumption is 3 W.

| (a) | Discuss in brief how the short circuit power and subthreshold leakage power dissipate in the device. | [3] |
| --- | --- | --- |
| (b) | Calculate the activity factor and switching frequency for each type of transistors. | [2] |
| (c) | Calculate the dynamic power and static power consumption. | [6] |
| (d) | Verify the statement that “By lowering the device speed by 10% we can meet the power consumption limit”. | [4] |