R2>en

R2#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#ip dhcp excluded-address 192.168.10.1 192.168.10.10

R2(config)#ip dhcp excluded-address 192.168.30.1 192.168.30.10

R2(config)#ip dhcp pool R1-LAN

R2(dhcp-config)#network 192.168.10.0 255.255.255.0

R2(dhcp-config)#default-router 192.168.10.1

R2(dhcp-config)#dns-server 192.168.20.254

R2(dhcp-config)#exit

R2(config)#ip dhcp pool R3-LAN

R2(dhcp-config)#network 192.168.30.0 255.255.255.0

R2(dhcp-config)#default-router 192.168.30.1

R2(dhcp-config)#dns-server 192.168.20.254

R2(dhcp-config)#exit

R1>en

R1#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface g0/0

R1(config-if)#ip helper-address 10.1.1.2

R1(config-if)#exit

R1(config)#

R3>en

R3#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#interface g0/0

R3(config-if)#ip helper-address 10.2.2.2

R3(config-if)#exit

R3(config)#

R2>en

R2#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface g0/1

R2(config-if)#ip address dhcp

R2(config-if)#no shutdown

R2(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

R2(config-if)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

R2#

%DHCP-6-ADDRESS_ASSIGN: Interface GigabitEthernet0/1 assigned DHCP address 209.165.200.231, mask 255.255.255.224, hostname R2

%DUAL-5-NBRCHANGE: IP-EIGRP 1: Neighbor 209.165.200.225 (GigabitEthernet0/1) is up: new adjacency

R2#show ip dhcp binding

IP address       Client-ID/              Lease expiration        Type

Hardware address

192.168.10.11    0002.4AA5.1470           --                     Automatic

192.168.30.11    0004.9A97.2535           --                     Automatic

R2#