Edge_Router>en

Edge_Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Edge_Router(config)#ip route 0.0.0.0 0.0.0.0 s0/0/0

%Default route without gateway, if not a point-to-point interface, may impact performance

Edge_Router(config)#ip route 0.0.0.0 0.0.0.0 s0/0/1 5

Edge_Router(config)#end

Edge_Router#

%SYS-5-CONFIG_I: Configured from console by console

Edge_Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C 10.10.10.0/30 is directly connected, Serial0/0/0

L 10.10.10.2/32 is directly connected, Serial0/0/0

C 10.10.10.4/30 is directly connected, Serial0/0/1

L 10.10.10.6/32 is directly connected, Serial0/0/1

192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.10.0/24 is directly connected, GigabitEthernet0/0

L 192.168.10.1/32 is directly connected, GigabitEthernet0/0

192.168.11.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.11.0/24 is directly connected, GigabitEthernet0/1

L 192.168.11.1/32 is directly connected, GigabitEthernet0/1

S* 0.0.0.0/0 is directly connected, Serial0/0/0

Edge_Router# conf t

Enter configuration commands, one per line. End with CNTL/Z.

Edge_Router(config)#int s0/0/0

Edge_Router(config-if)#shut down

^

% Invalid input detected at '^' marker.

Edge_Router(config-if)#shutdown

Edge_Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0/0, changed state to administratively down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to down

Edge_Router(config-if)#show ip route

^

% Invalid input detected at '^' marker.

Edge_Router(config-if)#end

Edge_Router#

%SYS-5-CONFIG_I: Configured from console by console

Edge_Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C 10.10.10.4/30 is directly connected, Serial0/0/1

L 10.10.10.6/32 is directly connected, Serial0/0/1

192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.10.0/24 is directly connected, GigabitEthernet0/0

L 192.168.10.1/32 is directly connected, GigabitEthernet0/0

192.168.11.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.11.0/24 is directly connected, GigabitEthernet0/1

L 192.168.11.1/32 is directly connected, GigabitEthernet0/1

S* 0.0.0.0/0 is directly connected, Serial0/0/1

Edge_Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Edge_Router(config)#int s0/0/0

Edge_Router(config-if)#no shut

Edge_Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0/0, changed state to up

Edge_Router(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to up

Edge_Router(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to up

Edge_Router(config-if)#exit

Edge_Router(config)#

Edge_Router(config)#ipv6 route ::/0 2001:DB8:A:2::1 5

Edge_Router(config)#